# Network Routing Static, RIP and Wireless

## 1. Network Topology Overview

- Router0 (R1)
  - IP: 192.168.2.1
  - Connected to Switch0 via 192.168.1.1
  - PCs: 192.168.1.2 (PC0), 192.168.1.3 (PC1)

- Router1 (R2)
  - IP: 192.168.2.2
  - Connected to Switch1 via 192.168.3.1
  - PCs: 192.168.3.2 (PC2), 192.168.3.3 (PC3)

## 2. Static Routing Configuration Steps

### On Router0 (R1):

Router> enable
Router# configure terminal
Router(config)# ip route 192.168.3.0 255.255.255.0 192.168.2.2
Router(config)# exit

### On Router1 (R2):

Router> enable
Router# configure terminal
Router(config)# ip route 192.168.1.0 255.255.255.0 192.168.2.1
Router(config)# exit

## 3. RIP Routing Configuration Steps

### On Both Routers (R1 and R2):

Router> enable
Router# configure terminal
Router(config)# router rip
Router(config-router)# version 2
Router(config-router)# no auto-summary
Router(config-router)# network 192.168.1.0
Router(config-router)# network 192.168.2.0
Router(config-router)# network 192.168.3.0
Router(config-router)# exit

💡 Use 'show ip route' to verify routing tables.

## 4. Wireless Routing Setup in Cisco Packet Tracer

Objective: Connect wireless devices to a network using a wireless router.

### Steps:

1. Add Wireless Router:
   - From the device list, drag a Wireless Router (e.g., Linksys) to the workspace.

2. Configure Wireless Router:
   - Click on the router → Go to GUI tab.
   - Set SSID (e.g., HomeNet).
   - Set Security Mode (e.g., WPA2 Personal).
   - Set Password (e.g., cisco123).

3. Add Wireless PCs:
   - Drag Laptop or PC with wireless capability.
   - Click on the PC → Desktop → PC Wireless → Connect to HomeNet.

4. Assign IP Addresses:
   - Either use DHCP from the router or assign static IPs manually.

5. Test Connectivity:
   - Use ping from one wireless PC to another or to a wired PC to verify routing.