# Networking and Linux Tasks Documentation

## Task 1: Connect Two Laptops

Connect Laptop 1 and Laptop 2 using a cross-over Ethernet cable or through a switch.
Assign IP addresses:
 - Laptop 1: 192.168.1.10
 - Laptop 2: 192.168.1.12
Subnet Mask: 255.255.255.0
Verify connectivity using the ping command from each laptop.

## Task 2: Connect 3 Laptops via Switch

Connect all three laptops to a switch using straight-through Ethernet cables.
Assign IP addresses:
 - Laptop 1: 192.168.1.10
 - Laptop 2: 192.168.1.11
 - Laptop 3: 192.168.1.12
Subnet Mask: 255.255.255.0
Test connectivity by pinging each laptop from the others.

## Task 3: Network with 4 PCs and 2 Laptops

Connect all devices to a switch using Ethernet cables.
Assign private IP addresses:
 - Dev Team PCs:
    * Dev-PC1: 192.168.1.101
    * Dev-PC2: 192.168.1.102
    * Dev-PC3: 192.168.1.103
 - DBA PC:
    * DBA-PC: 192.168.1.104
 - Tester Laptops:
    * Tester-Laptop1: 192.168.1.105
    * Tester-Laptop2: 192.168.1.106
Subnet Mask: 255.255.255.0
Label devices accordingly and verify connectivity using ping.

## Task 4: Install and Start httpd Service in Linux

Steps to install and start Apache HTTP Server (httpd):
1. Open terminal.
2. Install httpd:
   sudo dnf install httpd -y
3. Start the service:
   sudo systemctl start httpd
4. Enable service on boot:
   sudo systemctl enable httpd
5. Verify installation:
   Access http://localhost or http://<IP>
6. Check service status:
   sudo systemctl status httpd