# Cisco Packet Tracer Network Configuration Documentation

## Task 1: Static IP Configuration with Router, Switch, and 3 PCs

Steps:

- Create a router and assign a static IP (192.168.1.1/24) to its FastEthernet0/0 interface.

- Create a switch and connect it to the router.

- Connect three PCs to the switch.

- Configure each PC with a static IP and set the router IP as the default gateway.

Example CLI commands on router:

Router> enable
Router# configure terminal
Router(config)# interface FastEthernet0/0
Router(config-if)# ip address 192.168.1.1 255.255.255.0
Router(config-if)# no shutdown

## Task 2: Static Network Configuration with Router, Switch, and 3 PCs

Steps:

- Create a router and assign a static IP.

- Attach a switch to the router.

- Connect three PCs to the switch.

- Configure static IPs on each PC.

## Task 3: Find Network Details Using Command Prompt

Steps:

- Open Command Prompt on your system.

- Type the command: ipconfig

- Note the IPv4 address and subnet mask.

## Task 4: DHCP Network Configuration with Router, Switch, Server, and 5 PCs

Steps:

- Create a router and assign IP to its interface.

- Attach a switch to the router.

- Connect a server and five PCs to the switch.

- Configure the server as a DHCP server.

- Set PCs to obtain IP automatically.

## Task 5: VLAN Configuration for HR and DEV Departments

Steps:

- Create a router and attach a switch.

- Create two VLANs: VLAN 10 for HR and VLAN 20 for DEV.

- Assign HR1 and HR2 to VLAN 10, DEV1 and DEV2 to VLAN 20.

- Configure switch ports for respective VLANs.

- Ensure HR devices can communicate with each other and DEV devices can communicate with each other.

- Prevent communication between HR and DEV VLANs.

Example CLI commands on switch:

Switch> enable
Switch# configure terminal
Switch(config)# vlan 10
Switch(config-vlan)# name HR
Switch(config)# vlan 20
Switch(config-vlan)# name DEV
Switch(config)# interface FastEthernet0/1
Switch(config-if)# switchport mode access
Switch(config-if)# switchport access vlan 10
Switch(config)# interface FastEthernet0/2
Switch(config-if)# switchport mode access
Switch(config-if)# switchport access vlan 10
Switch(config)# interface FastEthernet0/3
Switch(config-if)# switchport mode access
Switch(config-if)# switchport access vlan 20
Switch(config)# interface FastEthernet0/4
Switch(config-if)# switchport mode access
Switch(config-if)# switchport access vlan 20